Quilcom THING

Design

Back in 2017 I made my Quilcom PMS (Physical Modelling Synthesiser) and I’ve since had a few requests to make a version which was easier to program. I also had a suggestion to make a synth along the lines of Reason’s Objekt synth, so I decided to combine the two ideas. The THING is not a copy of Objekt in any way, and there are significant differences, but the paradigm is somewhat similar. I should add that I’m not sure it is actually easier to program than the PMS!

Instead of conventional oscillators, we have resonant DSP modules. These require some sort of excitation to start producing sound. The MODAL resonators produce sinewaves which have little trace of the excitation signal when they start. The THING resonators rely on tuned delay lines with feedback to sustain the decaying signal. These resonators are heavily influenced by the nature of the excitation signal, since the excitation sound is circulated repeatedly at the frequency set. These very enhanced resonators were designed by Martin Vicanek and their performance far exceeds that of conventional Karplus-Strong algorithms. These were not available when I made the original PMS.

Overview

The whole column down the left side contains exciters of various types. The main top panel has 6 views for various sound production and processing functions, selected with the buttons above. The MIXER mixes and processes the audio from the resonators and exciters.

The Preset Manager contains a lot of conventional instrument sounds and a range of “Fantasy” presets. These can be thought of as templates to get you started, by selecting something that relates to what you want. Each template can have infinite variations adjusted. This type of synthesis lends itself better to some instrument sounds than others, so some will be more convincing than others.

EXCITERS

All the exciters can be turned on/off with the orange LED switch. They can be routed according to the 3 buttons: MO is MODALS, T1 is THING 1 and T2 is THING 2.

PULSE

An excitation pulse is a one-off shaped pulse. The LENGTH of the pulse affects the spectrum of the T1 and T2 sounds, so a longer pulse will give more bass to the sound. The very short CLICK, which triggers the shaped pulse, can be mixed in to give a sharper start to the sound. HARDNESS affects the shape of the pulse, to give higher emphasis to the initial sound spectrum produced. VEL adjusts the velocity response of the HARDNESS. This allows to provide a spectrum based on how hard the simulated instrument is excited. When VEL is at max, velocity control range is maximum, and when reduced, the range is less. At minimum, the HARDNESS is always maximum. The overall LEVEL of the pulse can be adjusted and the effect of velocity on the level can be set with the VEL knob alongside the LEVEL knob.

Depending on settings, the sounds may vary slightly on repeated playing. Greater variation between subsequent sounds will happen when VARY is turned ON.

Please note that the settings can interact and, other than LEVEL, have little effect on the MODAL spectrum.

When DIFFUSE is turned ON, the 3 knobs below will become available. Diffusion is when the shaped pulse is mixed with enveloped noise to blur the excitation. This could be used for a sliding hand-tap on a drum for example. TIME sets the duration of the added noise. POSITION adjusts when in time the shaped pulse is added to the noise. Fully down means the pulse starts when the noise starts. Fully up means the pulse comes at the end of the noise. Somewhere in-between is most useful. MIX sets the balance between the noise and the pulse.

NOISE

A burst of noise, or a sustained noise excitation, is set with the simple ADSR envelope generator. LEVEL can be set and VEL sets the velocity response. The onset of the ADSR envelope can be adjusted with the DELAY knob. This can be used to add noise after the initial PULSE for example to create a gong-type sound.

The COLOURED selector allows you to choose between various noise types as follows:

WHITE, COLOURED, CRACKLE, NOISE PULSE, RAND PULSE, PULSE, LP NOISE and COMB.

Some of these have an adjustment available which will always use the knob showing FREQ for noise colour in the image above. The label will change according to the function of the knob.

For those types of noise that can respond to key position, if you turn KBD ON, the adjustable parameter will have a pre-determined key-tracking set and the adjustable knob will dim.

PITCHED

PITCHED is a special excitation mode. Each resonator in the THING 1 and THING 2 generators will have a local oscillator to provide excitation of that resonator, when the PITCHED module is turned ON. The waveform can be either SINE or PULSE. For both waveforms, the SHAPE can be adjusted. For PULSE, the SHAPE knob adjusts the pulse width. The SINE waveform can be adjusted towards a smoothed sawtooth. The tuning is based on the individual resonators pitch settings, but can be shifted in octaves with the -1 OCT selector. The onset of the PITCHED signal can be adjusted with the DELAY knob. LEVEL and VELOCITY are available for the PITCHED excitation signal.

IN

This panel allows you to use an external audio signal to excite the resonators selected. A stereo input signal is converted to mono internally. The panel is not present on the “na” plugins (no audio).

To make use of this, you need to route the audio signal as a Send to the track containing the THING plugin, and not to the main DAW output. To hear any sound, you need to play notes on the MIDI keyboard, and those notes will be excited by the incoming audio. You can add audio IN excitation to other exciter signals of course. This allows you to use the plugin a bit like a vocoder, or to excite percussive sounds by tapping a microphone and so on. I’ve provided 3 preset templates; INPUT DEMO 1 to 3. These are not on the “na” plugin versions.

The LEVEL knob simply allows you to adjust the audio input level.

Tip: With a microphone input used for excitation, I suggest you listen on headphones to avoid feedback!

Sound generators

MODAL

Modal synthesis is essentially additive. Sinewaves at different frequencies are added together in different proportions to create more complex sounds. This MODAL module relies on a set of resonating complex filters which need to be excited to produce sound. They only produce sound at the set frequencies and so have a very limited spectral response to the shaped PULSE or NOISE excitation. The whole panel can be muted using the orange LED switch alongside the MODAL tab at the top.

There are 8 identical resonators provided. The ON row of switches turns their resonators on/off. The KBD row of switches turns keyboard tracking on/off. When turned off, the frequencies are static. The T buttons on the right toggle all the row’s buttons on/off.

The FREQ edit boxes set the pitch ratios based on the keyboard notes pressed, assuming the corresponding KBD switch is ON. So, a ratio of 2.000 will give a pitch exactly 1 octave above the base pitch played.

If the KBD switch is OFF, the frequency ratio is static (fixed) and based on Middle C (note 60), irrespective of what notes are played. So, if the ratio is set to 2.000, C5 (72) will always sound when any note is pressed.

You can click on an edit box and enter the ratio directly, or you can use the up/down arrow icons to drag with the mouse. If you double-click the arrow icon, the default ratio of 1 is set. If you hold down <SHIFT> while dragging, you get much finer control adjustment.  The R button at the end of the row sets all the ratios to 1.

The RING knob on the left sets the base decay time of all the resonators, and this has a wide range going from near instant to very long. The top row of sliders sets the RATIO of each resonator to the RING knob’s setting. So, if you have adjusted a RING time of 2 seconds, a slider set half way down will be approximately 1 second. These sliders allow you to set different decay times for each resonator, so the timbre will change as the overall sound decays. The ALL up/down arrow icon allows you to adjust all the RATIO sliders at once.

The LEVEL sliders allow you to adjust the level of each resonator. They are linear, so halfway down will be 50% amplitude. The ALL up/down arrow icon allows you to adjust all the LEVEL sliders at once.

PITCH

This section’s knobs set the pitch offsets for this module panel only (not the whole synth). The DETUNE knob provides a pitch offset for pairs of resonators, such that one will increase and other decrease in pitch to keep the average pitch constant.

VOL ENV

This basic ADSR envelope controls the volume of the whole module’s output mix. For excitation resonators it’s unusual to have this. I decided to include this so you could shape the volume envelope as you wish. That means you could, for example, have a percussive piano sound but increase the attack time to give a soft start to the sound in spite of needing a fast shaped pulse to initially excite the resonators. This ADSR can shorten the ring time by reducing D and R settings, and the maximum time available will be indicated on the knob readouts. This maximum time will be influenced by the RING times adjusted on the RING knob.

FOLDER

Each resonator has its own wave folder to turn its sinewave into a much brighter sound. To turn on the FOLDER for each resonator, use the row of FOLDER buttons. The T button at the end of the row will toggle all the FOLDER on/off buttons.

Wave folders are level sensitive, so the amount of folding is adjusted with the DRIVE knob. The SYMM knob adjusts the symmetry between positive-going folding and negative-going. This will affect the spectrum of the folded waveform. The BRIGHT knob adjusts the brightness of the folded signal and is a simple low pass filter. The AMNT knob adjusts the amount of folded signal added to the dry sinewave. The VOL ENV knob adjusts the amount of envelope control over the folded volume before it’s added to the dry sinewave. For maximum envelope swing, set the AMNT knob to minimum and the VOL ENV knob to maximum.

THING 1 and THING 2

THING 1 and THING 2 (T1 and T2) are identical. They can be muted with the orange LED switch alongside the tab at the top.

These modules make use of enhanced tuned digital delays which are excited from one or more EXCITER sources. The system itself has a lot in common with the MODAL panel.

There are 8 identical resonators provided. The ON row of switches turns their resonators on/off. The KBD row of switches turns keyboard tracking on/off. When turned off, the frequencies are static. The T buttons on the right toggle all the row’s buttons on/off.

The FREQ edit boxes set the pitch ratios based on the keyboard notes pressed, assuming the corresponding KBD switch is ON. So, a ratio of 2.000 will give a pitch exactly 1 octave above the base pitch played.

If the KBD switch is OFF, the frequency ratio is static (fixed) and based on Middle C (note 60), irrespective of what notes are played. So, if the ratio is set to 2.000, C5 (72) will always sound when any note is pressed.

You can click on an edit box and enter the ratio directly, or you can use the up/down arrow icons to drag with the mouse. If you double-click the arrow icon, the default ratio of 1 is set. If you hold down <SHIFT> while dragging, you get much finer control adjustment.  The R button at the end of the row sets all the ratios to 1.

The RING knob on the left sets the base decay time of all the resonators, and this has a wide range going from near instant to very long. The top row of sliders sets the RATIO of each resonator to the RING knob’s setting. So, if you have adjusted a RING time of 2 seconds, a slider set half way down will be approximately 1 second. These sliders allow you to set different decay times for each resonator, so the timbre will change as the overall sound decays. The ALL up/down arrow icon allows you to adjust all the RATIO sliders at once.

The LEVEL sliders allow you to adjust the level of each resonator. They are linear, so halfway down will be 50% amplitude. The ALL up/down arrow icon allows you to adjust all the LEVEL sliders at once.

The POSITION knob simulates the strike position of the exciter signal.

MAIN view:

The SEMI and FINE knobs set the pitch offsets for this module panel only (not the whole synth). The DETUNE knob provides a pitch offset for pairs of resonators, such that one will increase and other decrease in pitch to keep the average pitch constant.

The COUPLING knob works on adjacent pairs of resonators and feeds a curated signal from the righthand resonator to the lefthand excitation input to give a sound very much dependent on various settings. For example, there needs to be a difference in pitch values between the pairs to give any effect at all.

The BRIGHT knob is a simple low pass filter, provided to give a smoother sound if required.

The basic ADSR envelope controls the volume of the whole module’s output mix. For excitation resonators it’s unusual to have this. I decided to include this so you could shape the volume envelope as you wish. That means you could, for example, have a percussive piano sound but increase the attack time to give a soft start to the sound in spite of needing a fast shaped pulse to initially excite the resonators. This ADSR can shorten the ring time by reducing D and R settings, and the maximum time available will be indicated on the knob readouts. This maximum time will be influenced by the RING times adjusted on the RING knob. When the TRK button in ON, the Decay and Release times will be reduced for higher notes.

EXTRAS view

When EXTRAS is chosen a further range of controls is visible, as above.

The SITAR section can introduce a simulation of a jawari bridge to give a “twangy” character to the sound. LEVEL sets the amount of twang added to the dry signal. The THRESH knob sets the threshold above which the sound is produced. For maximum twang at all volumes, set the THRESH to maximum.

The BUZZ section aims to simulate collisions of a moving string (or whatever) with a fixed part of an instrument. The THRESH and LEVEL knobs work the same as in the SITAR section above. The DENSITY adjusts the number of collisions per unit time which will vary with amplitude.

Each resonator has its own FM DSP. This is simply a curated version of the output signal of each individual resonator fed back into its frequency control system. The AMOUNT knob adjusts the gain of this signal, and thus the modulation index. The effect is more noticeable on some settings than others. When PITCH is turned ON, the FM feedback becomes unipolar and thus dynamically affects the base pitch of the resonator. This is really only useful as a weird effect!

I’ve provided various filter systems in the resonators’ feedback (sustain) loop. Some of these will affect the base pitch because the loop delay will change at different frequencies. When the filter selector shows OFF, there is no filter in the loop and, thanks to Martin Vicanek, the pitch is true over a very wide musical range. The following filters are available:

APF (all pass filter), LPF (low pass filter), HPF (high pass filter), CHAOS, RING M (ring modulator) POWER and LIMIT. When any of these is selected, the ADJUST knob becomes available. This knob will then adjust the relevant parameter of whichever filter is chosen. I could spend a lot of time trying to describe the effect of these filters, but I prefer you to just check them out! Lazy or what?

PATH

In order to keep the T1 and T2 generator panels the same, and tidier, I needed to create a view especially for Exciter INPUT ROUTING. T1 INPUT mixes between the chosen Exciter(s) and the output from the MODAL generator. T2 INPUT mixes between, the chosen Exciter(s) and the output of T2. This gives a huge range of possibilities for chaining the generators to a greater or lesser extent.

Tip: some routings and settings can cause very loud sounds, so I suggest you turn the main VOLUME down considerably when experimenting.

MISC

You will see from the section labels that there are several functions on the MISC view panel.

GLOBAL

The selector showing 32 VOICES allow you to choose the maximum number of voices which can sound at any time.

SEMI and FINE adjust the offset tuning for the whole synth.

The SHRINK knob can reduce the pitch range played by the MIDI keyboard and interacts with the SEMI and FINE knobs on this panel and the 3 generator panels. This SHRINK control will reduce the pitch span above and below middle C (60). At minimum the keyboard will play true to pitch. At maximum, all notes will be middle C (depending on the other tuning knobs). In-between settings will give a reduced pitch span.

The BEND knob sets the maximum range of the pitch bend wheel, in semitones. When SMOOTH is selected the bend is smooth. If this is switched to STEPS, the bend will go up or down in semitone steps. This STEPS mode can be useful for keyed glissando, like on a saxophone.

When GLIDE is turned on, the TIME knob adjusts the portamento (glide) time taken to go from a note to the next one held down (or still sounding) and stolen. This means it works best when MONO is selected. If you choose 2 VOICES you can achieve a kind of limited polyphonic portamento when played appropriately. If you choose 4 VOICES, you can use the sustain pedal to hold notes for stealing and gliding.

PITCH ENV

The PITCH ENV can automatically bend any new note and affects the whole synth. TIME affects the duration of the bend. The RANGE knob is bipolar so no bend happens when set half way (default – double click). This Bipolar operation allows you to bend up or down to a new note’s final pitch. The VELOCITY knob adjusts the range of bend according to how hard the key is pressed, so a firmer strike will cause more bending. At maximum, key velocity has full control and at minimum there is no velocity control, and the RANGE is then always as set.

This simple polyphonic LFO allows you to modulate pitch (vibrato), amplitude (tremolo) and stereo pan position.

There are 3 ways to set the depth of modulation: KNOB for manual or automation (– use the CONTROL knob)), MWHL for modwheel depth or A-TOUCH for MIDI keyboard channel aftertouch (nice if you have it!). SPEED sets the rate of the LFO.

The VIBRATO knob sets the maximum excursion of the pitch modulation. UP-DOWN is symmetrical pitch modulation (e.g. violin style). This can be switched to UP ONLY (e.g. guitar style).

The TREMOLO knob is bipolar, so can be set to match the vibrato phase or go in the opposite phase. When set centrally, there is no tremolo.

PAN uses the LFO to create a “special” kind of left-right panning, assuming the MIXER (see later) is adjusted to create a stereo field. This special panning is designed to pan between full stereo, as generated, and mono.

SWELL

SWELL is a volume control that affects the level going into the FX section. This allows for expression to be added to any sound. The selector showing BREATH chooses the control source from the following CCs:

MODWHEEL, BREATH, FOOTPEDAL, VOLUME, EXPRESSION, RIBBON, SUSTAIN and FREE. When FREE is selected you can enter any CC number into the edit box. The VOLUME knob moves according to the controller position. When SWELL isn’t needed, the VOLUME knob is still active for manual adjustment or automation, so would normally be left set to maximum.

CONTROLLER 1 and 2

CONTROLLER 1 and 2 are identical. You can choose from a range of control sources as follows:

MODWHEEL, BREATH (as shown), FOOTPEDAL, VOLUME, EXPRESSION, RIBBON, SUSTAIN and FREE. When FREE is selected you can enter any CC number into the edit box.

When BLACK is turned ON, a signal is created only when any black note on the MIDI keyboard is played. The BREATH selector is then dimmed. This allows you to set a different adjustment for black notes to give, for example, a shorter RING (decay) time. One example of this would be to create open and closed hi-hat simulation by playing white notes and black notes.

To select a target or targets for the control signal, turn the labelled buttons ON.

Most of the small knobs below the target switches are bipolar and interact with the matching controls on those targets. The ZERO button sets the bipolar knobs to the central zero position. You can also double click these knobs individually to zero them.

MO LEVEL

These target the level sliders on resonators 7 and 8 on the MODAL generator.

RING ALL

This targets the RING time (decay) of all the generator panels. This is useful for setting different times when black keys are played for example.

T1 & T2 POSITION

This targets the strike POSITION on T1 and T2 generators.

T1 & T2 ADJUST FB

This targets the Feedback filter adjustment on T1 and T2 generators, assuming a filter with adjustment is selected on the targets.

MO FOLDER

AMNT targets the amount of FOLDER drive on the MODAL panel, assuming at least one resonator is set to route to the FOLDER section.

BRIGHT targets the brightness setting of the FOLDER.

EXCITERS

NOISE targets the amplitude of the NOISE exciter, assuming it’s turned ON.

PULSE targets the amplitude of the PULSE exciter, assuming it’s turned ON.

FX

I’ve provided 6 effect modules. These are chained in series as per the routing arrows. To turn the whole FX panel OFF/ON use the orange LED switch alongside the FX tab at the top. Individual effect modules can be turned ON/OFF with their orange LED switches.

DRIVE

DRIVE is a simple oversampled saturation and distortion module. GAIN sets the amount of drive and TONE sets the tone of the distortion. MIX adjusts the balance between the incoming dry signal and the distorted signal.

SOUNDBOX

This module can be used to add a range of resonances like you might get from an acoustic instrument’s sound box. It can add authenticity to an acoustic simulation. SIZE is analogous to the overall capacity of the box, going from small at minimum to large at maximum. The RES knob determines how much of the modified spectrum is added to the dry signal. At higher settings it’s analogous to the stiffness of the box construction.  Adjusting these 2 knobs is a case of listening and tweaking, but be aware that the sound will be affected by the notes played, so some may become over-emphasised. The LEVEL knob adjusts the amount of modified sound added to the incoming dry sound.

Tip: automating the SIZE knob can give a type of phasing sound, since comb filters are employed to give several peaks in the spectrum.

EQ

This is a simple half-octave graphic eq. You can drag across the sliders to easily create an EQ shape. When you hover over a slider its frequency will be displayed. When you drag on a slider the +/- dB will be displayed. Halfway (as shown) will be 0dB change. The R button resets all the sliders to 0dB. To easily adjust just one chosen slider, hold down the <Shift> key while dragging.

CHORUS

This makes use of two 4-phase delays, each quad delay being independent for left and right stereo channels. This arrangement provides for stereo creation and/or enhancement, since the Left and Right base delays and rates can be set independently and can be different. DEPTH adjusts both Left and Right modulation amounts together. The MIX knob sets the balance between the effect and the incoming dry signal.

ECHO

This is a simple stereo delay echo module. The selector showing 1/2 allows you to choose the BPM ratio for syncing to your DAW. If FREE is selected, the TIME knob becomes available for you to freely set your delay time. The SHIFT knob can provide a symmetrical offset to the delay times for Left and Right channels. So, one channel will increase the delay time and the other will decrease it, keeping the overall time impression fixed. This gives the option to create and/or modify the stereo field. At minimum, both Left and Right channel delays are identical.

FBK adjusts the feedback around the delays and thus controls the number of fading repeats. The DAMP knob adjusts the amount of filtering in the feedback loop and can be LPF (high frequency reduces per repeat) or HPF (low frequency reduces per repeat).

The MIX knob adjusts the balance between the echo signal and the incoming dry signal.

MVERB 7B

This is my favourite reverb, created by the remarkable Martin Vicanek.

PRE-DLY adjusts an optional pre-delay time. This holds off the signal before it gets into the reverb engine, and can create the illusion of being close to the source in a reverberant space. TAIL adjusts the time taken for the reverb tail to reach -60dB. DAMP adjusts the damping effect of a more absorbent space, reducing the high frequency content in the tail as it decays. WIDTH sets the stereo width of the reverb tail (not the dry signal). MIX adjusts the balance between the reverb and the incoming dry signal.

When FREEZE is turned ON, the tail stops decaying. In this situation, new sounds are not fed into the reverb engine.

The selector showing NO GATE can be changed to GATE 1 or GATE 2. These two gate times allow you to gate the reverb, which can create a Phil Collins gated drum sound for example.

MIXER

The N&P EQ (Noise & Pulse equalisation) knobs allow you to emphasise a static boost to a narrow part of the exciter spectrum. When the GAIN is at minimum no boost is added. When the GAIN is increased, the FREQ knob is available and adjusts the part of the spectrum to be boosted.

The sliders EXCITER, MODAL, THING 1 and THING 2 allow you to set the relative levels of the 4 sources. These are linear, such that a setting halfway down will reduce the level by 50%. Adding in some EXCITER signal could be used to add a nice click to a percussive sound for example.

By default, each generator creates a stereo signal based on MIDI note position on the keyboard. So lowest notes are panned left and highest notes panned right linearly across a 5-octave range. The STEREO slider adjusts the width of the mix, so at minimum it’s mono and at maximum it’s wide. Half way is stereo as created.

The LO CUT slider will reduce the high frequencies in the mix when raised above minimum.

TOOLS

The readout at the top shows the current number of voices sounding. If you want to clear the whole synth, click on this readout. This might be useful if you want to kill sounds with a very long RING time so you can make changes. If the sustain pedal is used a lot, it’s possible to occasionally get stuck voices. I’ve incorporated an auto-clear function for this situation so you just have to wait a couple of seconds, or you can clear the voice count manually. Please note this kills any effect currently sounding and clears the MIDI system.

Certain FB FILTER selections can offset the tuning. To manually adjust the base frequency, it’s useful to have a reference pitch. I’ve provided a 440 Hz (A4 69) reference signal for comparison and this is always available whether a note is played or not. The SHAPE knob adds distortion to the base sinewave which might help with the comparison. LEVEL adjusts the amplitude of this reference tone.

Preset Manager

I’ve included quite a few instrument presets which I think will be useful as templates or starting points for you to edit, to save you starting from scratch every time. The synth can also produce some wacky sounds which are explored in a limited way by the “Fantasy” presets.

At the top of the preset manager is the section where you select the preset by clicking on the preset name or paging though them using the arrow buttons. The synth is silenced and reset when a preset is changed.

The MENU selector is where you operate on presets and banks. You can save, load, copy or paste presets, or save and load a bank from this menu.

All changes made to any settings will be stored with the DAW song file unless the switch UNLOCKED is changed to LOCKED. This locking feature is to avoid losing settings if you just want to mess with editing but want to keep the original default parameters.

The RENAME button allows you to name or rename a preset, providing the preset manager is UNLOCKED. Otherwise, the RENAME button is dimmed.

At the bottom is a free text area for adding comments to the preset. These comments are saved with the song, and the preset if you save it, providing the preset manager is UNLOCKED. Please be aware that you shouldn’t use a carriage return (Enter) in this text because the system won’t store any text after that. Also please be aware that when you RENAME a preset this text will clear, so if you want to keep it and just rename the preset, highlight the text, copy it then paste back in after you’ve renamed.

VOLUME

The output VOLUME knob has 2 bar meters which indicate average peak values. If the signal goes outside +/-1 even very briefly, the inner circle will turn red for 1 second to indicate clipping. If you need accuracy or a different type of level indication, please rely on your DAW’s meter system.