Quilcom FLEXATONE

Design

Someone suggested I have a go at a Flexatone, so, not knowing what one even is, I looked into it and bought one for fun, and to help me get the sound and behaviour better. First sold in America by Playatone, this 1923 invention is a novel percussion instrument mostly used as a sound effect in old cartoons and spooky films. You shake it to make the balls strike the metal plate and flex the plate with your thumb to change the pitch and timbre.

In use

The operation is really quite simple. If you click on the  button you’ll see reminder text as follows:

Ref Middle C is C4

Play using keys C4 to C5. The shake speed increases from C4 to C5.

The last note played is taken. When holding down more than one note, the first note played determines the velocity. This value is held when playing legato.

The CONTROL value, typically from the modwheel, determines the pitch of the spring plate.

The FM for vibrato can optionally be gated by the sustain (hold) pedal.

For the details, please read the User Guide!

The CONTROL panel provides the parameter for changing the pitch and, optionally, the vibrato rate. Normally the selector would choose MODWHEEL as shown. You select from the following sources for the CONTROL signal:

MODWHEEL, BREATH, FOOT PEDAL, VOLUME, EXPRESSION, RIBBON, PITCHBEND, FREE

Apart from PITCHBEND the others are Continuous Controllers (CC). If you choose FREE, you can enter any CC number you wish in the edit box.

I think MODWHEEL is better for pitch control than the PITCHBEND wheel in this plugin, because you have a greater movement of the wheel for finer and easier pitch control.

The MANUAL knob shows the current CONTROL source value, but you can also operate it with the mouse and automate it.

On my real Flexatone, I found the hammers will often bounce against the plate, so the sound is not a simple repeating sound but has a kind of random effect. The BOUNCE knob adjusts the bounce parameter and creates a much more realistic behaviour. It will vary with the shake rate selected by the note played on the keyboard. This pseudo-random effect is because the hammer-spiring setup has its own static vibration frequency, irrespective of the shake rate and pitch.

The panel adjusts the MIN and MAX pitches when bending the virtual plate. If you double-click the knobs, the defaults will be set which match my real Flexatone. The range of these frequencies goes beyond what’s possible on real instruments.

The classic sound of a Flexatone relies on applying mechanical vibrato to the bend pressure while, or just after, shaking. This can quite easily be achieved by operating the modwheel to produce the exact pitch “envelope shape” you want.

In order to make it easier, and to provide vibrato rates which would be impossible with your thumb or modwheel, I’ve included a vibrato LFO. I tried various waveforms but Sine was by far the best.

When KNOB is selected, as shown, the FREQ knob controls the vibrato speed. The following sources can be chosen:

KNOB Manual (or automated) LFO frequency adjustment.

SHAKE The shake speed note played will alter the LFO frequency.

VEL the velocity of the shake note played will alter the LFO frequency.

CC The CONTROL source, e.g. MODWHEEL, will alter the LFO frequency.

ATOUCH Channel aftertouch pressure will alter the LFO frequency.

ENV – An envelope follower will start at a high LFO frequency and drop as the volume fades.

ENV + An envelope follower will start at a low LFO frequency and increase as the volume fades.

The RANGE knob adjusts the depth (amount) of the vibrato frequency excursion.

The switch showing PEDAL can be changed to ON.

PEDAL means you can use the sustain (hold) pedal to bring in the vibrato at will. This can also be achieved by sending CC64 as on/off.

ON simply means the vibrato is always present and not affected by any incoming control signals.

The DECAY knob adjusts the decay time of 4 resonant delay line filters (designed by Martin Vicanek). At minimum, the sound is very damped and at maximum it last longer than my real Flexatone. You could automate this knob for rhythmic creativity (damp… un-damp).

The BRIGHT knob controls the spectrum of the sound and is very much biased towards the initial attack sound.

The CLANG knob is something I added for fun, and can add a dissonant lower pitched impact sound. It’s not there on a real Flexatone!

I’ve included my simple recorder to this plugin. This allows you to record sounds and save the recording as a regular WAV file. One reason you may want to do this is because the sounds generated can have random components. By recording a WAV and putting it into the DAW as part of a rhythm, you’ll get the exact same sound repeating.

The TIME sets the estimated recording time in 2 adjustment ranges. To get ready for recording, click the ARM button. The recording will start when you play the keyboard. A red progress bar runs at the top of the module. After the bar has stopped, click on the SAVE button to save the WAV file. It can take a few seconds to save it because it has to be compiled first. If you’re not satisfied you can just repeat the process.

The X button will stop the recording and clear the buffer, so there’ll be nothing to save.

I’ve provided a few presets to demonstrate some different settings.

At the top of the preset manager is the section where you select the preset by clicking on the preset name or paging though them using the arrow buttons. The synth is silenced and reset when a preset is changed.

The MENU selector is where you operate on presets and banks. You can save, load, copy or paste presets, or save and load a bank from this menu.

All changes made to any settings will be stored with the DAW song file unless the switch UNLOCKED is changed to LOCKED. This locking feature is to avoid losing settings if you just want to mess with editing but want to keep the original default parameters.

The RENAME button allows you to name or rename a preset, providing the preset manager is UNLOCKED. Otherwise, the RENAME button is dimmed.

At the bottom is a free text area for adding comments to the preset. These comments are saved with the song, and the preset if you save it, providing the preset manager is UNLOCKED. Please be aware that you shouldn’t use a carriage return (Enter) in this text because the system won’t store any text after that. Also please be aware that when you RENAME a preset this text will clear, so if you want to keep it and just rename the preset, highlight the text, copy it then paste back in after you’ve renamed.

The main output VOLUME has a bar graph for left and right (even though this plugin is mono!). They indicate average peak levels. If you want alternative bar graphs, please use your DAWs metering.

Even a very short peak level excursion beyond +/- 1 will cause clipping and the knob’s centre will turn red and hold for 1 second.